Field Programming the Kenwood TK-270 or TK-370 handhelds.

Press and hold (light) & (dial) while turning on radio.

Hold until (SEL) appears on the display.

Pressing (SCN) allows you to program the following

1.Monitor Key-----------

Off = off

3 = Squelch off momentary

2.Scan Key--------------

Co = carrier operate

3.Dial Key--------------

On = enable

4.TA key----------------

TA = talkaround

5.Lo Key----------------

On

6.Priority--------------

None

7.Fixed Priority Channel

Turn selector to channel (1)

8.Look back time A------

Turn selector to time in seconds 0.5

9.Look back time B------

Turn selector to time in seconds 2.0

10.Revert Channel-------

2 = Last Call

11.Tx Dwell time--------

Turn selector to time in seconds 0.3

12.Drop out delay-------

Turn selector to time in seconds 0.3

13.Time out timer-------

Turn selector to time in seconds 60

14.Tot Pre-alert--------

Turn selector to time in seconds. off

15.Tot Rekey time-------

Turn selector to time in seconds. off

16.Tot reset time-------

Turn selector to time in seconds. off

17.Squelch Level--------

Turn selector to Squelch (1)

18.Beep tone------------

On

19.Signalling-----------

And

20.Battery Save---------

On

21.Selectable QT--------

On

22.Clr to transpond-----

Off

23.Del/ Add-------------

On

Pressing (Dial) allows programming of:

DTMF

24.digit time-----------

Turn selector to time in milliseconds. 50

25.inter digit time-----

Turn selector to time in milliseconds. 50

26.First digit time-----

Turn selector to time in milliseconds. 50

27.Rise time------------

Turn selector to time in milliseconds. 100

28.Rise time with QT----

Turn selector to time in milliseconds. 100

29.PTT ID---------------

Off

30.Dial ID--------------

Off

31.Connect ID-----------

32.Disconnect ID--------

33.# of DTMF keys on Keypad

16

34.DTMF hold time-------

On

35.Store & send---------

On

36.D key assignment

Turn selector to “D” or time in seconds for pause

37.Dtmf Signaling-------

1 = code squelch

38.Intermediate Code----

“F”

39.Group Code-----------

OFF

40.Auto Reset time------

Turn selector for time in Seconds. 10

41.Call Alert/ Transpond

Off

42.Transpond------------

Off

Pressing (TA) will allow you to program Frequencies

Use selector to choose channel

Press PTT

Enter Rx frequency using:------------------

- the Selector Knob to change kHz;

- the Selector Knob + (light) to change MHz.

If no channel is Shown press the (LO) button.

*Press PTT

Enter Rx CTCSS code-----

*Press PTT

Enter Tx Frequency as above--------------

*Press PTT

Enter Tx CTCSS code-----

*Press PTT

1.Option Signal---------

1 = DTMF

2.PTT ID----------------

Off

3.Add/ Del—Add

4.Busy Tx--Off

5.Clock frequency shift. Off

6.Power-----------------

Hi

Repeat for the other 31 channels

<end>